STANDARD OPERATING PROCEDURE

Smart Factory Dashboard

| Document No. | SFD-SOP-001 |
| --- | --- |
| Version | 1.0 |
| Effective Date | April 22, 2026 |
| Department | Manufacturing / Production |
| Prepared By | Dashboard Development Team |
| Classification | Internal Use Only |

# 1. Purpose

This Standard Operating Procedure (SOP) defines the correct procedures for operating the Smart Factory Dashboard application. The dashboard integrates three data systems — AGV (Automated Guided Vehicle) log monitoring, OEE (Overall Equipment Effectiveness) performance tracking, and AOI (Automated Optical Inspection) quality control — into a unified real-time management interface.

The purpose of this document is to ensure all users can load data, interpret dashboard indicators, and respond to alerts in a consistent and accurate manner.

# 2. Scope

This SOP applies to all personnel who use the Smart Factory Dashboard, including:

- Production Supervisors and Line Managers

- Manufacturing Engineers and Process Engineers

- Quality Control Personnel

- Factory Directors and Senior Management

# 3. System Overview

The Smart Factory Dashboard is a desktop application built with Python and PyQt5. It processes and visualizes data from three independent systems:

| Module | Data Source | Purpose |
| --- | --- | --- |
| AGV | Log files (.txt / .log) | Monitor robot vehicle traffic, task completion, and connectivity health |
| OEE | Excel / CSV files (.xls / .xlsx / .csv) | Track equipment Availability, Performance, and Quality by production line |
| AOI | Image folder (PASS / FAIL images) | Measure optical inspection pass rates per production lane |

# 4. Installation and Launch

## 4.1 System Requirements

| Requirement | Specification |
| --- | --- |
| Operating System | Windows 10 / Windows 11 (64-bit) |
| Processor | Intel Core i5 or equivalent, 2.0 GHz minimum |
| Memory | 4 GB RAM minimum, 8 GB recommended |
| Disk Space | 500 MB free space |
| Display | 1920 x 1080 resolution or higher |
| Python (source only) | Python 3.10 or later |

## 4.2 Launching the Application

If using the compiled executable:

- Locate the file SmartFactory.exe in the provided folder.

- Double-click SmartFactory.exe to launch.

- The dashboard will open in full-screen mode automatically.

If running from source code:

- Open a Command Prompt in the project folder.

- Run the command: python main.py

- Wait for the dashboard window to appear.

# 5. Loading Data

All three data modules (AGV, OEE, AOI) are loaded independently. Only one loading task may run at a time. Buttons are disabled while a task is in progress.

## 5.1 Loading AGV Logs

- Click the Load AGV Logs button in the System & Data panel.

- A file selection dialog will appear. Navigate to the folder containing AGV log files.

- Select one or more .txt or .log files. Multiple files can be selected by holding Ctrl.

- Click Open. Processing begins automatically.

- The status bar at the top-right will show progress. When complete, it displays Done.

The dashboard will update the following panels:

- AGV Tasks: total task count, completion ratio, peak hour

- MES Server: number of failed report transmissions to MES

- Vehicle Health: offline disconnect count per vehicle

- Hourly Traffic chart: task volume distribution across the day

- Top Stations chart: most frequently visited stations

## 5.2 Loading OEE Files

- Click the Load OEE File button.

- Select one or more Excel or CSV files (.xls, .xlsx, or .csv).

- Click Open. The system processes and merges all selected files.

- Once loaded, use the Date Filter dropdown to view data for a specific date, or leave it on All to view the full period.

The OEE file must contain the following columns:

| Column | Description |
| --- | --- |
| 日 (Date) | Date of the record |
| 樓層 (Floor) | Floor identifier, e.g., F4 or F5 |
| 綫 (Line) | Production line name, e.g., 4A, 4B, 5G |
| OEE | Overall Equipment Effectiveness value (percentage) |
| A | Availability rate (percentage) |
| P | Performance rate (percentage) |
| Q | Quality rate (percentage) |

## 5.3 Loading AOI Data

AOI data is loaded from a folder containing inspection result images. Image filenames must include either ALL PASS or fail (case-insensitive) to be classified.

Supported folder structures:

- Option A (Parent folder): Select a parent folder that contains sub-folders named after each production lane (e.g., 4A, 4B, Revlon1-3). The system will automatically match each sub-folder to the corresponding lane.

- Option B (Direct folder): Select the lane folder directly (e.g., select the 4A folder). The system detects the lane name from the folder name.

Supported lane folder names:

| Display Name | Accepted Folder Names |
| --- | --- |
| 4A | 4A |
| 4B | 4B |
| 4C | 4C |
| Revlon 1&3 | Revlon1-3, Revlon13, Revlon1&3 |
| Revlon 2&4 | Revlon2-4, Revlon24, Revlon2&4 |
| ICX | ICX, icx |
| CCX | CCX, ccx |
| Giftbox | Giftbox, giftbox, GiftBox |

To load multiple folders (accumulate data across shifts or dates):

- Click Select AOI Folder and select the first folder.

- Click Select AOI Folder again and select the next folder. Results are accumulated.

- To clear all AOI data and start fresh, click the Reset AOI button.

# 6. Reading the Dashboard

## 6.1 AGV Section

| Indicator | Meaning |
| --- | --- |
| Task count (top number) | Total number of task records extracted from the loaded log files |
| Done / Started (e.g., 108/118) | Tasks confirmed completed vs. tasks initiated. A gap indicates unconfirmed tasks. |
| MES failures (large red number) | Number of times the AGV system failed to report to the MES server. Any value above 0 indicates a connectivity issue. |
| Vehicle Health bars | Horizontal bars showing offline disconnect events per vehicle. Red = critical (over 100), Yellow = caution (20-100), Green = normal. |
| Red dot on traffic chart | Marks the peak hour with the highest task volume. |

## 6.2 OEE Section

| Indicator | Meaning |
| --- | --- |
| F4 avg / F5 avg | Average OEE across all production lines on Floor 4 and Floor 5 respectively |
| Grouped bar chart | OEE value per production line. Orange bars = F4 lines, Yellow bars = F5 lines. |
| A / P / Q bars | Breakdown of OEE components. Performance (P) below 50% indicates machines running slower than designed speed. |
| Insight text | Automatic analysis: identifies the worst-performing line and flags F5 if below 20%. |

## 6.3 AOI Section

| Indicator | Meaning |
| --- | --- |
| Progress bars (green/red) | Each bar represents one production lane. Green segment = pass count, Red segment = fail count. Grey = no data loaded for that lane. |
| Percentage label | Pass rate for that lane. Green text = at or above 90% threshold. Red text = below threshold. |
| Overall pass rate (large number) | Aggregated pass rate across all lanes with data. |
| Warning message | Identifies the lane with the lowest pass rate and recommends action. |

# 7. Alert Thresholds and Response Actions

| Alert | Condition | Required Action |
| --- | --- | --- |
| MES Server Failure | MES failure count > 0 | Notify IT/Network team. Verify server at 10.175.26.58:8092 is reachable. |
| Vehicle Critical | Vehicle offline count > 100 | Stop the vehicle for inspection. Check WiFi antenna and hardware connections. |
| Vehicle Caution | Vehicle offline count 20-100 | Monitor closely. Schedule inspection at next maintenance window. |
| OEE Low F5 | F5 average OEE < 20% | Check maintenance schedule for Floor 5 equipment. Verify 5H line status. |
| OEE Bottleneck | Performance (P) < 50% | Investigate machine speed settings. Equipment may be running below rated capacity. |
| AOI Lane Fail | Lane pass rate < 50% | Halt AOI inspection on that lane. Recalibrate AOI machine before resuming. |
| AOI Lane Warning | Lane pass rate 50-89% | Review recent inspection images. Check product quality and machine alignment. |
| Unconfirmed Tasks | Done count < Started count | Investigate log files for incomplete task records. Check AGV system logs. |

# 8. Language Settings

The dashboard supports three display languages. Language can be changed at any time without restarting the application or reloading data.

| Button | Language |
| --- | --- |
| VI | Vietnamese |
| EN | English |
| ZH (Chinese) | Simplified Chinese |

To change language: click the corresponding language button in the top-right area of the title bar. All labels, section headings, chart titles, and insight messages will update immediately.

# 9. Troubleshooting

| Issue | Likely Cause | Resolution |
| --- | --- | --- |
| Application fails to launch | Missing Python or dependencies | Run build_exe.bat to reinstall dependencies, or reinstall SmartFactory.exe |
| OEE file not loading | File format not supported or columns missing | Verify the file contains columns: 日, 樓層, 綫, OEE, A, P, Q |
| AOI bars show no data | Folder name does not match expected lane names | Rename the folder to match an accepted name (see Section 5.3) |
| Chinese characters appear as boxes in charts | CJK font not installed on system | Install Microsoft YaHei font. Right-click the font file and select Install. |
| Warning: file size not multiple of sector size | Old-format .xls file (harmless) | Warning can be ignored. Data is still read correctly. |
| Charts not updating after loading | Data loading still in progress | Wait for the status bar to show Done before reading charts. |

# 10. File and Folder Naming Conventions

## 10.1 AGV Log Files

Log files should follow the naming pattern: LogYYYYMMDDHH.txt (e.g., Log2026040110.txt). Multiple log files from different hours or shifts can be loaded simultaneously.

## 10.2 AOI Image Files

Image filenames must contain one of the following keywords:

- ALL PASS (case-insensitive) — counted as a passing inspection result

- fail (case-insensitive) — counted as a failing inspection result

Images not containing either keyword are ignored. Example valid filenames:

- PVN28481HHG ALL PASS 20260316153934.jpg

- 20260316153516_fail.jpg

# 11. Revision History

| Version | Date | Description | Author |
| --- | --- | --- | --- |
| 1.0 | 4/22/2026 | Initial release | Development Team |

# Document Approval

| Role | Name | Signature | Date |
| --- | --- | --- | --- |
| Prepared By |  |  |  |
| Reviewed By |  |  |  |
| Approved By |  |  |  |